IT Intro Crash Course

7 Essential Parts:

| Computer Case | Power Supply | Motherboard | Central Processing Unit / Processor SOCKET (PGA vs LGA) | Random Access Memory Stick (RAM) | Graphical Processing Unit | Solid State Drive / Hard Disk Drive |
| --- | --- | --- | --- | --- | --- | --- |
| Plastic Box Housing Everything Protect Sensitive Hardware From Dust, Water, EMF, etc.. | Plugs Into Outlet Regulates Power AC Adapter Power Source | Circuit Board All of Components Plug into It Provide Interconnectivity! | CPU Processor “Does Things” Very Low Data Storage “Center Part Of The Brains Of Computer” | Memory Data / Files Storage Small Storage Space Fast Access. “Double Bay Door Storage Shed” | GPU Video Card Computer In Itself | Disk Drive Storage Data/Files Storage Large Storage Access Slow Access “Massive Warehouse with Small Doorway” |
|  |  |  |  | 60 GB (in process) Run more/fancier programs | Fast Approximate Inverse Square Root | 1000 GB (at rest) Have more installed Have more videos/pictures/files |

| Cooling System / LED | Network Interface Card / Wifi-Card | Capacitor / Resistor / | PATA / SATA / USB / PCI(express) | NorthBus (with heatsink) / SouthBus | IDE / AGP/ Connectors of Integrated Peripherals | BIOS / CMOS / ATX Power Connector |
| --- | --- | --- | --- | --- | --- | --- |

## Networking Hardware

This next video describes the core components of network hardware.

### The Essentials

| - Hub | - Bridge | - Switch | - Router |
| --- | --- | --- | --- |
| -Layer 1 Device (no knowledge of addresses) Blindly diffuse all data it receives 1 Collision Domain Half-Duplex (can’t send and receive data at the same time without causing Data Collision) Wasted Bandwidth (by sending unnecessary data to wrong users) Security Risks (all host receives everyone data, making it easier for malicious actors. Old and Outdated (Replaced by switches) | -Introduce as a sort of solution to Hubs. -Layer 2 Device (Understands and Learns MAC Addresses) -Segment Lans -2 Collision Domains (data can be sent at same time) - Fewer than 2 Ports -Look at Source & Destination MAC Address to know where to forward the data, discard the data | Hub and Bridge combined: Generates MAC ADDRESS TABLE (MAC of EACH device and it’s CONNECTED PORT) Layer 2 Device (Layer 3 Switches with Router Functionality) Full Duplex: Send and Receive Data at the same time Multiple Collision Domains: Each port has its own Collision Domains Saves Bandwidth from Learning MAC Addresses Increase Security: Only destined device receives data forwarded by the Switch. | Doorway Out of Your Internal Network into the Outside World Network. Routes Traffic Between Networks. Utilizes IP Addresses To Forward Data to Appropriate Host / Server. Highly Configurable Layer 3 Device (Layer 3 Addresses: IP Addresses) |

| Server | Dedicated Computer Equipped and Designed To Serve Resources/ Application/Data/ Services to Client Computers: Linux, Windows Server, macOs Server (ROBUST & STABLE) Able to handle thousands on concurrent connections | Server Services Desktop, Workstations, etc.. Centralized Machine Connected to Multiple Clients Over The Internet Or On a Local Area Network (LAN). Hot-swappable HARD DRIVE in RAID configuration. Multiple Redundant Power Supplies. Servers need Server Operating System. Web Server Email Server Database Server | Desktops Can Be Set-up As Servers, But Not Suitable Due Limited Hardware & Software Capacities | Servers Must Be Built/Designed with Robust Hardware & Software To Ensure Business Continuity | Intel Core i7 FOR DESKTOP Intel Xeon Processor FOR SERVERS (Fast and Able To Engage Multiple Processes Simultaneously) -Larger Cache -More Cores | CPU Servers Designed Accommodate Multiples Processors.Intel I-Core 7 DO not Function with ECC RAM. AMD Processor DO SUPORT ECC | Xeon Processors – ECC Ram (Error Correcting Code)Detects Of DATA was processed properly and CORRECT the data if need be. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Centers | Brains of The Internet. Engine of The Internet. | Rows after rows Of Servers Working Together TO Provide Access To DATA and SERVICES in the Internet. |  |  |  |  |  |  |  |

# IT Job Preparation

At SkillStorm, we look for candidates with a background in IT for our Cloud and Cyber Security roles. Most candidates who have earned entry-level IT certifications perform well in these roles (CompTIA A+, Network+, Linux+, and Security+). Here are a few concepts and terminology to review to prepare for an IT interview.

Make sure you know concise definitions for each bullet point

Check out “Professor Messer” on YouTube for additional videos

# Hardware

- CPU: Computer Processing Unit- A central processing unit, also called a central processor, main processor or just processor, is the electronic circuitry that executes instructions comprising a computer program. The CPU performs basic arithmetic, logic, controlling, and input/output operations specified by the instructions in the program

- RAM: Random Access Memory- RAM (Random Access Memory) is the hardware in a computing device where the operating system (OS), application programs and data in current use are kept so they can be quickly reached by the device's processor. RAM is the main memory in a computer.

- HDD: Hard Disk Drive- A hard disk drive (HDD), hard disk, hard drive, or fixed disk is an electro-mechanical data storage device that stores and retrieves digital data using magnetic storage with one or more rigid rapidly rotating platters coated with magnetic material.

- SSD: Solid State Drive- A solid-state drive is a solid-state storage device that uses integrated circuit assemblies to store data persistently, typically using flash memory, and functioning as secondary storage in the hierarchy of computer storage.

- Ethernet cabling: Mainly there are three types of ethernet cables used in LANs i.e., Coaxial cables, Twisted Pair cables, and Fiber optic cables

Ethernet IEEE 802.3 Includes:
Ethernet introduction     Standards     Ethernet data frame structure     100Mbps Fast Ethernet     Gigabit Ethernet, 1GE     10 Gigabit Ethernet, 10GE     Single Pair Ethernet, SPE     Ethernet cables     How to buy Ethernet cables     Routers, hubs, switches - the differences     Ethernet switch     How to buy best Ethernet switch     Ethernet industrial switch     Power over Ethernet, PoE     Ethernet splitter     Carrier Ethernet     Ethernet Products Shopping Page

Ethernet is a set of technologies and protocols that are used primarily in LANs. It was first standardized in 1980s by IEEE 802.3 standard. IEEE 802.3 defines the physical layer and the medium access control (MAC) sub-layer of the data link layer for wired Ethernet networks.

This article is about the standards working group. For Ethernet frame data format, see Ethernet frame.

IEEE 802.3 is a working group and a collection of Institute of Electrical and Electronics Engineers (IEEE) standards produced by the working group defining the physical layer and data link layer's media access control (MAC) of wired Ethernet. This is generally a local area network (LAN) technology with some wide area network (WAN) applications. Physical connections are made between nodes and/or infrastructure devices (hubs, switches, routers) by various types of copper or fiber cable.

802.3 is a technology that supports the IEEE 802.1 network architecture.

802.3 also defines LAN access method using CSMA/CD.

There are many Ethernet cables that can be bought. Often these cables are supplied free with equipment that uses Ethernet connectivity in some way or another.

There are several different varieties and categories of Ethernet cable that can be obtained: speed variations, crossover cables, Cat 5, Cat 5e, Cat6, Cat 6a, Cat 7etc..

Normally Ethernet cables will be bought and there is no major need to understand what is inside or on the connectors, although it can be both interesting and helpful on some occasions. Even so, an understanding of the different types of Ethernet cable and the maximum lengths that should be used is helpful.

The commonly used network cables: Cat 5, Cat 5e, Cat 6, Cat 6a, Cat7 all have different levels of performance, and therefore to is necessary to buy or select the right cable for the right application.

These network cables are used for connecting a variety of network elements from Ethernet switches and Ethernet routers to computers, servers and other network items - if there is an Ethernet interface, they can be connected using Ethernet cables.

In fact Ethernet cables of all types form the backbone of virtually all local area networks regardless of the network topology used, and as such they are of immense importance and interest.

## Ethernet cable basics

The Ethernet cables for connectivity in most office and home local area networks rely on twisted wire pairs within an overall cable - Cat 5, Cat 6 and Cat 7 all used this format.

Twisting the wires together enables the currents to balance, i.e in one wire the current is moving in one direction, and in the other wire of the pair the current is going in the other, enabling the overall fields around the twisted pair to cancel.

In this way, data can be transmitted over considerable lengths without the need for undue precautions.

As several twisted pairs are contained within a particular data network cable, the number of twisted per unit length is arranged to be different for each pair - the rate being based on prime numbers so that no two twists ever align. This reduces crosstalk within the cable.

The Ethernet cables are available in a variety of lengths as patch cables, or the cable itself is available for incorporating into systems, buildings, etc. The terminations can then be made to the required connector using a crimp tool. These network cables are available in a variety of lengths - long Ethernet cables are available, some of the longest being up to 75 meters.

Earlier network cables were unshielded, but later ones were shielded to improve the performance. For example, an unshielded twisted pair, UTP, cable may be satisfactory for a short run between a computer and router, but a foil shielded cable, FTP, is best longer runs or where the cable passes through areas of high electrical noise.

There are different methods that can be used for shielding Ethernet cables. The most common is to place a shield around each twisted pair. This not only provides shielding for the cable externally, but also reduces crosstalk between the internal twisted pairs as well.

Manufacturers can further enhance the performance by placing shielding around all the wires in the cable just under the cable sheath. There are different codes used to indicate the differs types of shielding:

- U/UTP - Unshielded cable, unshielded twisted pairs

- F/UTP - Foil shielded cable, unshielded twisted pairs

- U/FTP - Unshielded cable, foil shielded twisted pairs

- S/FTP - braided shielded cable, foil shielded twisted pairs

Where: TP = twisted pair, U = unshielded, F = foil shielded, S = braided shielding.

A further difference within the Ethernet cables whether Cat 5, Cat 5e, Cat 6, Cat 6e, or Cat 7 can be whether solid or stranded wires are used within the cable. As the description implies, a solid cable uses a single piece of copper for the electrical conductor within each wire of the cable whilst stranded wire uses a series of copper strands twisted together.

Although when buying a patch cable, it may not be necessary to know this, when installing a long cable run it may be important as each type is slightly more suitable for different applications.

- Stranded cable:   This type of wire is more flexible, and it is more applicable for Ethernet cables where the cable may be moved - often it is idea for patch leads at desks or general connections to PCs, etc. where some movement may be needed and expected.

- Solid cable:   Solid cable is not as flexible as the stranded type, but it is also more durable. This makes it best for use in permanent installations like cable installations under floors, embedded in walls and the like.

## Categories for Ethernet cables

A variety of different cables are available for Ethernet and other telecommunications and networking applications. These network cables that are described by their different Ethernet cable categories, e.g. Cat 5 cables, Cat 6 cables, Cat 7 and Cat 8etc, which are often recognized by the TIA (telecommunications Industries Association) and they are summarized below:

- Cat-1:     This is not recognized by the TIA/EIA. It is the form of wiring that is used for standard telephone (POTS) wiring, or for ISDN (ISDN stands for Integrated Services Digital Network. It's a set of communication standards that uses digital transmission to make phone calls, video calls, transmit data and other network services over the circuits of the traditional PSTN (Public Switched Telephone Network). ISDN was introduced in 1986 by BT.)

- Cat-2:     This is not recognized by the TIA/EIA. It was the form of wiring that was used for 4Mbit/s token ring networks.

- Cat-3:     This cable is defined in TIA/EIA-568-B. It is used for data networks employing frequencies up to 16 MHz It was popular for use with 10 Mbps Ethernet networks (100Base-T), but has now been superseded by Cat-5 cable.

- Cat-4:     This cable is not recognized by the TIA/EIA. However, it can be used for networks carrying frequencies up to 20 MHz. It was often used on 16Mbps token ring networks.

- Cat-5:     This is not recognized by the TIA/EIA. This is the network cable that is widely used for 100Base-T and 1000Base-T networks as it provides performance to allow data at 100 Mbps and slightly more (125 MHz for 1000Base-T) Ethernet. The Cat 5 cable superseded the Cat 3 version and for several years it became the standard for Ethernet cabling. Cat 5 cable is now obsolete and therefore it is not recommended for new installations.

- Cat 5 cable uses twisted pairs to prevent internal crosstalk, XT and crosstalk to external wires, AXT.

- Although not standardized, the Cat 5 cable normally uses 1.5 - 2 twists per centimeter.

- Cat-5e:     This form of cable is recognized by the TIA/EIA and is defined in TIA/EIA-568, being last revised in 2001. It has a slightly higher frequency specification than Cat-5 cable as the performance extends up to 125 Mbps.

- Cat-5e: can be used for 100Base-T and 1000Base-t (Gigabit Ethernet). Cat 5e standard for Cat 5 enhanced and it is a form of Cat 5 cable manufactured to higher specifications although physically the same as Cat 5. It is tested to a higher specification to ensure it can perform at the higher data speeds. The twisted pairs within the network cables tend to have the same level of twisting as the Cat 5 cables.

- Cat-6:     This cable is defined in TIA/EIA-568-B provides a significant improvement in performance over Cat5 and Cat 5e. During manufacture Cat 6 cables are more tightly wound than either Cat 5 or Cat 5e and they often have an outer foil or braided shielding.

- The shielding protects the twisted pairs of wires inside the Ethernet cable, helping to prevent crosstalk and noise interference. Cat-6 cables can technically support speeds up to 10 Gbps but can only do so for up to 55 meters - even so this makes them relatively long Ethernet cables.

- The Cat 6 Ethernet cables generally have 2+ twists per cm and some may include a nylon spline to reduce crosstalk, although this is not actually required by the standard.

- Cat-6a:     The “a” in Cat 6a stands for “Augmented” and the standard was revised in 2008. The Cat 6a cables can support twice the maximum bandwidth and are capable of maintaining higher transmission speeds over longer network cable lengths. Cat 6a cables utilize shielded which is sufficient to all but eliminate crosstalk.

- However, this makes them less flexible than Cat 6 cable.

- Cat-7:     This is an informal number for ISO/IEC 11801 Class F cabling. It comprises four individually shielded pairs inside an overall shield. It is aimed at applications where transmission of frequencies up to 600 Mbps is required.

- Cat-8:     Cat 8 cables have now been released and provide a huge step up in data rate / bandwidth. Accordingly, these Cat 8 cables are generally more expensive than the older versions like Cat 6, or even Cat 7.

- Further descriptions of Cat-5 and Cat-5e cables are given below as these are widely used for Ethernet networking applications today.

| ETHERNET CABLE PERFORMANCE SUMMARY |  |  |  |
| --- | --- | --- | --- |
| CATEGORY | SHIELDING | MAX TRANSMISSION SPEED (AT 100 METERS) | MAX BANDWIDTH |
| Cat 3 | Unshielded | 10 Mbps | 16 MHz |
| Cat 5 | Unshielded | 10/100 Mbps | 100 MHz |
| Cat 5e | Unshielded | 1000 Mbps / 1 Gbps | 100 MHz |
| Cat 6 | Shielded or Unshielded | 1000 Mbps / 1 Gbps | >250 MHz |
| Cat 6a | Shielded | 10000 Mbps / 10 Gbps | 500 MHz |
| Cat 7 | Shielded | 10000 Mbps / 10 Gbps | 600 MHz |
| Cat 8 | Shielded | 25 Gbps or 40Gbps * | 2000 MHz |

- * 25 Gbps for Cat 8.1 and 40 Gbps for Cat 8.2.

## Ethernet cable connectors

- The RJ45, Registered Jack 45 connector is used almost universally as the physical connector used on Ethernet cables, and with networking cables in general.

- From Ethernet Category 3 right through to Cat 6, the RJ45 is the format that is used. Cat 7 Ethernet cables can be terminated with RJ45 connectors but specialised versions called GigaGate45 (GG45) are often used. Fortunately these are backwards compatible with the RJ45 so there is no need to have a completely new installation when migrating to Cat 7.

- Ethernet RJ45 connector on an Ethernet cable

- The RJ45 connector used on the ends of Ethernet cables are small plastic plugs with a retaining catch that can be released when the cable needs to be removed. The term plug refers to the male end of the connection on the network cable, and the jack refers to the port or female and normally located on the equipment.

- The RJ45 connector has eight pins that are spaced around 1 mm apart, and the wires are inserted and crimped to provide a reliable connection. The actual connector type is known as an 8P8C - eight position - eight contact.

- Strictly speaking the actual connector type should be reared to as an 8P8C, and the wiring pattern is RJ45, but in reality the term RJ45 for Ethernet cables is used almost universally.

## Ethercon connectors

- Although the RJ45 connector is ideal for most applications, it is not particularly robust. It is suitable for office, data-center and home connections where robustness is not an issue. However there are a number of scenarios where a more robust connector is needed - applications like audio visual, stage audio and lighting, etc where Ethernet is used need a much more robust connection.

- To address this problem the Ethercon connector is used. The Ethercon connector can be likened to a combination of an RJ45 with an XLR connector.

- As the Ethercon connector has a circular metal outer protection combined with the RG45 style inner, it provides the required connectivity with the robustness required for applications like stage and other audio visual areas where Ethernet connections are required.

- It is possible not only to obtain the Ethercon connectors on their own, but also ready made Ethercon leads as well.

## Ethernet cable pinout

- Although the wiring and the cable manufacture details may vary between the different cable categories, the basic connectivity remains the same. In this way Ethernet cables can be used reliably to make connections between items of equipment, etc.

- A summary of the signals carried and the relevant wires and connections is given in the table below:

| RJ-45 / CAT 5 / CAT 5E PINOUT & WIRING |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PIN NO | TELEPHONE | 10BASE-T | 100BASE-T | 1000BASE-T | POE MODE A | POE MODE B |
| 1 |  | +TX | +TD | +BI_DA | 48 V out |  |
| 2 |  | -TX | -TX | -BI_DA | 48 V out |  |
| 3 |  | +RX | +RX | +BI_DB | 48 V return |  |
| 4 | Ring |  |  | +BI_DC |  | 48 V out |
| 5 | Tip |  |  | -BI_DC |  | 48 V out |
| 6 |  | -RX | -RX | -BI_DB | 48 V return |  |
| 7 |  |  |  | +BI_DD |  | 48 V return |
| 8 |  |  |  | -BI_DD |  | 48 V return |

- In the table, TX is transmitted data, and RX is received data. BI_Dn is bi-directional data, A, B, C, and D.

- There are two standard RJ45 pinouts for the individual arrangement of the wire connections to the RJ45 connectors within an Ethernet cable: the T568A and T568B standards.

- One or other of the conventions should be followed, as this will ensure the required connectivity, although as it is just a color convention and the same wires and pairs are connected to the same pins, it does not matter which one is followed. In practice T568B is the more commonly used, and therefore it is probably best to use this one. The standard used may also be indicated on the sheath of ready made Ethernet cables.

| RJ-45 PINOUT & WIRING |  |  |
| --- | --- | --- |
| PIN | T568A | T568B |
| 1 | White with green stripe | White with orange stripe |
| 2 | Green | Orange |
| 3 | White with orange stripe | White with green stripe |
| 4 | Blue | Blue |
| 5 | White with blue stripe | White with blue stripe |
| 6 | Orange | Green |
| 7 | White with brown stripe | White with brown stripe |
| 8 | Brown | Brown |

- Ethernet cable with outer sheath stripped back to show the internal twisted pair wires

## Ethernet Cat 5 crossover cables

- There are a number of different configurations of cable that may be employed according to the equipment and the requirement. The most common type are the straight through cables which are wired in a 1 to 1 configuration. However Cat-5 crossover cables are also required on occasions.

- Typically a Cat-5 cable used to connect a computer (PC) to a switch will be a straight through cable. However if two computers or two switches are connected together then a Cat5 crossover cable is used.

- Many Ethernet interfaces in use today are able to detect the type of cable, whether it is a straight through or crossover cable, and they are able to adapt to the required format. This means that the requirement for Cat-5 crossover cables is less than it might otherwise be.

- When using Cat-5 Ethernet crossover cables, they are not marked with the fact that they are crossover cables. Accordingly it is often wise to mark them to avoid confusion later.

## Ethernet cable maximum lengths

- Often it is necessary to use long Ethernet cables, however there are limits to the distance over which the data can be reliably carried. When long Ethernet cables are used, additional noise is picked up, and the data eventually becomes distorted by the cable.

- A summary of Ethernet cables and their maximum operating lengths is given below:

| MAXIMUM LENGTHS FOR LONG ETHERNET CABLES |  |  |
| --- | --- | --- |
| SPECIFICATION | CABLE TYPE | MAXIMUM LENGTH |
| 10BaseT | Unshielded Twisted Pair | 100 metres |
| 10Base2 | Thin coaxial cable | 180 metres |
| 10Base5 | Thick coaxial cable | 500 metres |
| 10BaseF | Fibre optic cable | 2000 metres |
| 100BaseT | Unshielded twisted pair | 100 metres |
| 100BaseTX | Unshielded twisted pair | 220 metres |

- Lengths provided are generally accepted as the maximum but are not included in the IEEE standard.

### Check out the Electronics Notes:

### Essential Ethernet Equipment List.

- A variety of different cables of different types and Ethernet cable categories are seen connecting different Ethernet elements together - Ethernet routers, Ethernet switches, computers, network severs, etc. Cat5, Cat5e and Cat6 are all seen. These are often within a local area network of one form or another.

- The cables use the cost efficient but effective RJ45 patch connector and using this, these Ethernet cables are able to link or patch multiple items of Ethernet based equipment together.

- On some occasions where specialised connectivity is needed crossover cables may be required - these sold be clearly marked as such as they may get used elsewhere unknowingly and cause the connection not to work. Apart from this, using Cat 5, Cat5e, Cat 6, Cat 7 and now Cat 8 Ethernet cables is very easy and there are normally very few issues.

Patch Cable: A patch cable connects two electronic or optical devices to each other for signal routing. This is usually for network applications, to “patch” a signal from one hub, switch, or router to another hub, switch, or router. Use patch cables to carry a variety of signals, such as: telephone.

What is the difference between a patch cable and an Ethernet cable?

Ethernet cables are simply wires that comply with Ethernet protocols. These cables are made of copper or optical fiber. Patch cables are simply ethernet cables which have connectors at both ends.

And patch cables are often used for short distances in offices and wiring closets. Ethernet patch cable can link a computer to a network hub, router or Ethernet switch, which is useful to people constructing home computer networks.

- Network adapter: A network adapter is a piece of hardware that acts as the interface for a computer to a network. This way, computers can communicate across a network. The network adapter allows the device to communicate over the local area network (LAN), connecting to the internet or to other computers. Wireless network ...

- Modem: The sole purpose of the modem is to provide you with internet access. If you were to only have one internet-connected device with an Ethernet port (such as a desktop computer), you could connect the modem directly to your computer with no need for a router. Your modem is a box that connects your home network to the wider Internet. A router is a box that lets all of your wired and wireless devices use that Internet connection at once and also allows them to talk to one another without having to do so over the Internet.

- Router:  A router connects your devices to each other and, in hard-wired connection setups, to the modem. The router connects to your modem and then to your devices (laptops, smart TVs, printers, etc.) via either an Ethernet cable or, in the case of a wireless router, Wi-Fi signal.

- Switch: A network switch is networking hardware that connects devices on a computer network by using packet switching to receive and forward data to the destination device. A network switch is a multiport network bridge that uses MAC addresses to forward data at the data link layer of the OSI model. A network switch connects devices (such as computers, printers, wireless access points) in a network to each other, and allows them to 'talk' by exchanging data packets. Switches can be hardware devices that manage physical networks, as well as software-based virtual devices.

- Hub: A network hub is a node that broadcasts data to every computer or Ethernet-based device connected to it. A hub is less sophisticated than a switch, the latter of which can isolate data transmissions to specific devices. Network hubs are best suited for small, simple local area network (LAN) environments. An Ethernet hub, active hub, network hub, repeater hub, multiport repeater, or simply hub is a network hardware device for connecting multiple Ethernet devices together and making them act as a single network segment.

- Server:  In computing, a server is a piece of computer hardware or software that provides functionality for other programs or devices, called "clients". This architecture is called the client–server model. A server operating system, such as Windows Server or Linux, acts as the platform that enables applications to run. The operating system provides applications access to the hardware resources that they need and enables network connectivity. The application is what enables the server to do its job. On a local area network, a print server manages one or more printers, and prints files sent to it by client computers. Network servers (which manage network traffic) and file servers (which store and retrieve files for clients) are two more examples of servers.

- Database:  A database is an organized collection of structured information, or data, typically stored electronically in a computer system. A database is usually controlled by a database management system (DBMS). In computing, a database is an organized collection of data stored and accessed electronically. Small databases can be stored on a file system, while large databases are hosted on computer clusters or cloud storage. Structured Query Language (SQL). Hierarchical database systems. network database systems. object-oriented database systems. Below are some common types of databases: Centralized database. Cloud database. Commercial database. Internal Level or Physical level. Conceptual Level. External Level or View Level. While most non-techies have never heard of Google's Bigtable, they've probably used it. It is the database that runs Google's Internet search, Google Maps, YouTube, Gmail, and other products you've likely heard of. It's a big, powerful database that handles lots of different data types. Databases are widely divided into two major types or categories, namely, Relational or Sequence Databases and Non-relational or Non-sequence databases or No SQL databases. A database is a collection of information that is related. Access allows you to manage your information in one database file. Within Access there are four major objects: Tables, Queries, Forms and Reports.

- BIOS: Basic Input Output System (BIOS (basic input/output system) is the program a computer's microprocessor uses to start the computer system after it is powered on. It also manages data flow between the computer's operating system (OS) and attached devices, such as the hard disk, video adapter, keyboard, mouse and printer. In computing, BIOS is firmware used to provide runtime services for operating systems and programs and to perform hardware initialization during the booting process. The BIOS firmware comes pre-installed on an IBM PC or IBM PC compatible's system board and exists in UEFI-based systems too. Unified Extensible Firmware Interface (UEFI) computerworld.com

- Virtualization: One of the features included in Windows 10 is the ability to create virtual machines. A virtual machine is a packaged operating system that can run on top of a “host” operating system. Virtualization allows the same host to run multiple “guest” operating systems, and easily move virtual machines between hosts. In computing, virtualization or virtualization is the act of creating a virtual version of something at the same abstraction level, including virtual computer hardware platforms, storage devices, and computer network resources. Virtualization is technology that lets you create useful IT services using resources that are traditionally bound to hardware. It allows you to use a physical machine's full capacity by distributing its capabilities among many users or environments. Server virtualization is the process of using software to divide physical hardware into separate unique virtual servers. Read about the three main types of server virtualization: full-virtualization, para-virtualization, and OS-level virtualization.

- RAID: RAID stands for Redundant Array of Inexpensive Disks. That means that RAID is a way of logically putting multiple disks together into a single array. The idea then is that these disks working together will have the speed and/or reliability of a more expensive disk. Now, the exact speed and reliability you'll achieve from RAID depends on the type of RAID you're using.

- A Short Overview of Spinning Disk and Solid State Drives

- Spinning disk, mechanical hard drives, or Hard Disk Disks (HDDs) are typically chosen in situations where needs such as speed and performance fall second to cost. Due to physical limitations and the mechanical nature of many high speed moving parts contained in them, HDDs also have a relatively high failure rate compared to SSDs. RAID is meant to help alleviate both of these issues, depending on the RAID type you use. Typically, a mechanical hard drive has a 2.5% chance of failure each year of its operation. This has been proven by multiple reports and no specific manufacturer or model has a dramatic variation from that 2.5% rate. In short, if you value your data, you are going to need to implement some methodology to help protect it from drive failure.

- SSDs are typically chosen in situations where speed and performance take a priority to cost considerations. As they have no moving parts, their ability to both write and read data on them is significantly faster than on a HDD (at least 8-10x faster.) And their failure rate is roughly .5% during each year of is operation, which significantly reduces the risk compared to a spinning HDD.

- Because of the dramatic difference between the technologies of HDDs and SSDs, it’s important to state that some RAID implementations that are great for HDDs are not for SSDs, and vice versa.

- What Are the Types of RAID?

- RAID 0 (Striping)

- RAID 0 is taking any number of disks and merging them into one large volume. This will greatly increase speeds, as you're reading and writing from multiple disks at a time. An individual file can then use the speed and capacity of all the drives of the array. The downside to RAID 0 though is that it is NOT redundant. The loss of any individual disk will cause complete data loss. This RAID type is very much less reliable than having a single disk.

- There are rarely a situation where you should use RAID 0 in a server environment. You can use it for cache or other purposes where speed is important and reliability/data loss does not matter at all. But it should not be used for anything other than that. As an example, with the 2.5% annual failure rate of drives, if you have a 6 disk RAID 0 array, you've increased your annual risk of data loss to nearly 13.5%.

- RAID 1 (Mirroring)

- While RAID 1 is capable of a much more complicated configuration, almost every use case of RAID 1 is where you have a pair of identical disks identically mirror/copy the data equally across the drives in the array. The point of RAID 1 is primarily for redundancy. If you completely lose a drive, you can still stay up and running off the additional drive.

- In the event that either drive fails, you can then replace the broken drive with little to no downtime. RAID 1 also gives you the additional benefit of increased read performance, as data can be read off any of the drives in the array. The downsides are that you will have slightly higher write latency. Since the data needs to be written to both drives in the array, you'll only have the available capacity of a single drive while needing two drives.

- RAID 5/6 (Striping + Distributed Parity)

- RAID 5 requires the use of at least 3 drives (RAID 6 requires at least 4 drives). It takes the idea of RAID 0, and strips data across multiple drives to increase performance. But, it also adds the aspect of redundancy by distributing parity information across the disks. There are many technical resources out there on the Internet that can get down into the details as to how this actually happens. But in short, with RAID 5 you can lose one disk, and with RAID 6 you can lose two disks, and still maintain your operations and data.

- RAID 5 and 6 will get you significantly improved read performance. But write performance is largely dependent on the RAID controller used. For RAID 5 or 6, you will most certainly need a dedicated hardware controller. This is due to the need to calculate the parity data and write it across all the disks. RAID 5 and RAID 6 are often good options for standard web servers, file servers, and other general purpose systems where most of the transactions are reads, and get you a good value for your money. This is because you only need to purchase one additional drive for RAID 5 (or two additional drives for RAID 6) to add speed and redundancy.

- RAID 5 or RAID 6 is not the best choice for a heavy write environment, such as a database server, as it will likely hurt your overall performance.

- It is worth mentioning that in a RAID 5 or RAID 6 situation, if you lose a drive, you’re going to be seriusly sacrificing performance to keep your environment operational. Once you replace the failed drive, data will need to be rebuilt out of the parity information. This will take a significant amount of the total performance of the array. These rebuild times continue to grow more and more each year, as drives get larger and larger.

- RAID 10 (Mirroring + Striping)

- RAID 10 requires at least 4 drives and is a combination of RAID 1 (mirroring) and RAID 0 (striping). This will get you both increased speed and redundancy. This is often the recommended RAID level if you're looking for speed, but still need redundancy. In a four-drive configuration, two mirrored drives hold half of the striped data and another two mirror the other half of the data. This means you can lose any single drive, and then possibly even a 2nd drive, without losing any data. Just like RAID 1, you'll only have the capacity of half the drives, but you will see improved read and write performance. You will also have the fast rebuild time of RAID 1.

- When Should I Use RAID?

- RAID is extremely useful if uptime and availability are important to you or your business. Backups will help insure you from a catastrophic data loss. But, restoring large amounts of data, like when you experience a drive failure, can take many hours to perform. Those backups could be hours or days old, costing you all the data stored or changed since the last backup. RAID allows you to weather the failure of one or more drives without data loss and, in many cases, without any downtime.

- RAID is also useful if you are having disk IO issues, where applications are waiting on the disk to perform tasks. Going with RAID will provide you additional throughput by allowing you to read and write data from multiple drives instead of a single drive. Additionally, if you go with hardware RAID, the hardware RAID card will include additional memory to be used as cache, reducing the strain put on the physical hardware and increase overall performance.

- Note: We generally do not advise using a hardware RAID card for SSD volumes, as the additional cache isn’t necessary because of the speed of the SSDs themselves.

- What Type of RAID Should I Use?

- No RAID - Good if you are able to endure several hours of downtime and/or data loss due while you restore your site from backups.

- RAID 0 - Good if data is unimportant and can be lost, but performance is critical (such as with cache).

- RAID 1 - Good if you are looking to inexpensively gain additional data redundancy and/or read speeds. (This is a good base level for those looking to achieve high uptime and increase the performance of backups.)

- RAID 5/6 -  Good if you you have Web servers, high read environments, or extremely large storage arrays as a single object. This will perform worse than RAID 1 on writes. If your environment is write-heavy, or you don't need more space than is allowed on a disk with RAID 1, RAID 1 is likely a more effective option.

- RAID 10 - A good all-around solution that provides additional read and write speed as well as additional redundancy.

- Software vs Hardware?

- Software RAID

- Software RAID is an included option in all of Steadfast’s dedicated servers. This means there is NO cost for software RAID 1, and is highly recommended if you’re using local storage on a system. It is highly recommended that drives in a RAID array be of the same type and size.

- Software-based RAID will leverage some of the system’s computing power to manage the RAID configuration. If you’re looking to maximize performance of a system, such with a RAID 5 or 6 configuration, it’s best to use a hardware-based RAID card when you’re using standard HDDs.

- Hardware RAID

- Hardware-based RAID requires a dedicated controller installed in the server. Steadfast engineers will be happy to provide you with recommendations regarding which hardware RAID care is best for you that is based on what RAID configuration you want to have. A hardware based RAID card does all the management of the RAID array(s), providing logical disks to the system with no overheard on the part of the system itself. Additionally, hardware RAID can provide many different types of RAID configurations simultaneously to the system. This includes providing a RAID 1 array for the boot and application drive and a RAID-5 array for the large storage array.

- What Does RAID Not Do?

- RAID does not equate to 100% uptime. Nothing can. RAID is another tool in the toolbox meant to help minimize downtime and availability issues. There is still a risk of a RAID card failure, though that is significantly lower than a mechanical HDD drive failure.

- RAID does not replace backups. Nothing can replace a well-planned and frequently tested backup implementation!

- RAID will not protect you against data corruption, human error, or security issues. While it can protect you against a drive failure, there are innumerable reasons for keeping backups. So do not take RAID as a replacement for backups. If you don’t have backups in place, you’re not ready to consider RAID as an option.

- RAID does not necessarily allow you to dynamically increase the size of the array. If you need more disk space, you cannot simply add another drive to the array. You are likely going to have to start from scratch, rebuilding/reformatting the array. Luckily, Steadfast engineers are here to help you architect and execute whatever systems you need to keep your business running.

- RAID isn’t always the best option for virtualization and high-availability failover. In those circumstances, you will want to look at SAN solutions, which Steadfast also provides.

Study Video: https://www.youtube.com/watch?v=2eLe7uz-7CM

# Networking & Ports

- OSI Model: The open systems interconnection (OSI) model is a conceptual model created by the International Organization for Standardization which enables diverse communication systems to communicate using standard protocols. The Open Systems Interconnection model is a conceptual model that describes the universal standard of communication functions of a telecommunication system or computing system, without any regard to the system's underlying internal technology and specific protocol suites.

- HTTP (Port 80): On a Web server or Hypertext Transfer Protocol daemon, port 80 is the port that the server "listens to" or expects to receive from a Web client, assuming that the default was taken when the server was configured or set up. However, Port 80 provides an HTTP connection under TCP protocol. This port provides an unencrypted connection between the web browser and the web servers, which leaves the sensitive user data exposed to cybercriminals and may lead to severe data misuse.

- HTTPS (Port 443): Port 443 is a virtual port that computers use to divert network traffic. Billions of people across the globe use it every single day. Any web search you make, your computer connects with a server that hosts that information and fetches it for you. This connection is made via a port – either

HTTPS or HTTP port.

- SSH (Port 22): Secure Shell, Secure File Transfer Protocol, Secure Socket Layer: Secure sockets layer (SSL) is a networking protocol designed for securing connections between web clients and web servers over an insecure network, such as the internet.

- Replaced by Transport Security Layer: Transport Layer Security is a cryptographic protocol designed to provide communications security over a computer network. The protocol is widely used in applications such as email, instant messaging, and voice over IP, but its use in securing HTTPS remains the most publicly visible.

- FTP (Port 20 & 21): File Transfer Protocol- FTP is used for file transfers between one system and another, and it has several common use cases, including the following: Backup. FTP can be used by backup services or individual users to backup data from one location to a secured backup server running FTP services. The File Transfer Protocol is a standard communication protocol used for the transfer of computer files from a server to a client on a computer network. FTP is built on a client–server model architecture using separate control and data connections between the client and the server.

- Telnet (Port 23): Telnet is a client-server protocol, based on a reliable connection-oriented transport. Typically, this protocol is used to establish a connection to Transmission Control Protocol (TCP) port number 23, where a Telnet server application (telnetd) is listening.

- SMTP (Port 25): What is the SMTP port 25?

Port 25 is the default port for relaying email on the internet. However, you can specify any port number (Provided it is within the acceptable range of custom port numbers) for submitting emails to your SMTP server. There are special cases when you must use port 25 on your SMTP server. Is SMTP port 25 secure?

SMTP was designated to use port 25 in IETF Request For Comments (RFC) 821. IANA still recognizes Port 25 as the standard, default SMTP port. The port is no longer recognized by IANA. This port has secure according to the guidelines set out by the IETF.

- POP3 (Port 110):

Pop3 "post office protocol" is used by eMail clients for the retrieval of their eMail from designated eMail "post office" servers. Email Clients such as Microsoft Outlook, Netscape, Eudora, and many others, connect to port 110 of a remote eMail server, then use the pop3 protocol to retrieve their eMail.

- DNS (Port 53): DNS uses Port 53 which is nearly always open on systems, firewalls, and clients to transmit DNS queries. Rather than the more familiar Transmission Control Protocol (TCP) these queries use User Datagram Protocol (UDP) because of its low-latency, bandwidth and resource usage compared TCP-equivalent queries.

- Firewall: Technical control: In computing, a firewall is a network security system that monitors, and controls incoming and outgoing network traffic based on predetermined security rules. A firewall typically establishes a barrier between a trusted network and an untrusted network, such as the Interne

- TCP: TCP stands for Transmission Control Protocol a communications standard that enables application programs and computing devices to exchange messages over a network. It is designed to send packets across the internet and ensure the successful delivery of data and messages over networks. The Transmission Control Protocol is one of the main protocols of the Internet protocol suite. It originated in the initial network implementation in which it complemented the Internet Protocol. Therefore, the entire suite is commonly referred to as TCP/IP. Wikipedia

Developer(s): Vint Cerf and Bob Kahn

RFC(s): RFC 9293

Based on: Transmission Control Program

Introduction: 1974

- UDP: User Datagram Protocol (UDP) is a communications protocol that is primarily used to establish low-latency and loss-tolerating connections between applications on the internet. UDP speeds up transmissions by enabling the transfer of data before an agreement is provided by the receiving party. In computer networking, the User Datagram Protocol is one of the core members of the Internet protocol suite. With UDP, computer applications can send messages, in this case referred to as datagrams, to other hosts on an Internet Protocol network.

- VLAN: A virtual LAN (VLAN) is a logical overlay network that groups together a subset of devices that share a physical LAN, isolating the traffic for each group. A LAN is a group of computers or other devices in the same place -- e.g., the same building or campus -- that share the same physical network.

- IP: IP stands for "Internet Protocol," which is the set of rules governing the format of data sent via the internet or local network. In essence, IP addresses are the identifier that allows information to be sent between devices on a network: they contain location information and make devices accessible for communication.

- MAC: In the OSI reference model for networking, the MAC is a Layer 2 -- or data link layer -- device, and the MAC address is a Layer 2 address. In the current internet era, most devices are connected physically with Ethernet cables or wirelessly with Wi-Fi. Both methods use MAC addresses to identify a device on the network. A media access control address (MAC address) is a unique identifier assigned to a network interface controller (NIC) for use as a network address in communications within a network segment. This use is common in most IEEE 802 networking technologies, including Ethernet, Wi-Fi, and Bluetooth.

- Network Topology: Network topology is the arrangement of the elements of a communication network. Network topology can be used to define or describe the arrangement of various types of telecommunication networks, including command and control radio networks, industrial fieldbuses and computer networks.

- WiFi:

Study Video: https://www.youtube.com/watch?v=vrh0epPAC5w

# Operating Systems

- Linux and Windows

- Basic Linux Commands

- CD

- LS

- MV

- CP

- MKDIR

- RMDIR

- PWD

- CHMOD

Study Video: https://youtu.be/Wgi-OfbP2Gw?t=4212

# Security

- Authentication: the process or action of proving or showing something to be true, genuine, or valid.

"the prints will be stamped with his seal and accompanied by a letter of authentication"

  - COMPUTING

  - the process or action of verifying the identity of a user or process.

  - "user authentication for each device ensures that the individual using the device is recognized by the company".

Authentication is the act of proving an assertion, such as the identity of a computer system user. In contrast with identification, the act of indicating a person or thing's identity, authentication is the process of verifying that identity

- Authorization:  Authorization is a process by which a server determines if the client has permission to use a resource or access a file. Authorization is usually coupled with authentication so that the server has some concept of who the client is that is requesting access.

- CIA: The three letters in "CIA triad" stand for Confidentiality, Integrity, and Availability. The CIA triad is a common model that forms the basis for the development of security systems. They are used for finding vulnerabilities and methods for creating solutions.

- Social Engineering: In the context of information security, social engineering is the psychological manipulation of people into performing actions or divulging confidential information. This differs from social engineering within the social sciences, which does not concern the divulging of confidential information.

- Malware: Malware is any software intentionally designed to cause disruption to a computer, server, client, or computer network, leak private information, gain unauthorized access to information or systems, deprive users access to information or which unknowingly interferes with the user's computer security and privacy.

- DDoS: In computing, a denial-of-service attack is a cyber-attack in which the perpetrator seeks to make a machine or network resource unavailable to its intended users by temporarily or indefinitely disrupting services of a host connected to a network.

- Hot-site/Cold-site: A hot site is fully functional and allows immediate recovery from a disaster while a cold site only includes infrastructure but no technology until a disaster hits. A hot site is a commercial disaster recovery service that allows a business to continue computer and network operations in the event of a computer or equipment disaster. For example, if an enterprise's data center becomes inoperable, that enterprise can move all data processing operations to a hot site. A backup facility that has the necessary electrical and physical components of a computer facility, but does not have the computer equipment in place.

- Honeypot: A honeypot is a network-attached system set up as a decoy to lure cyber attackers and detect, deflect and study hacking attempts to gain unauthorized access to information systems.

- Multi-factor Authentication: Multi-factor authentication is an electronic authentication method in which a user is granted access to a website or application only after successfully presenting two or more pieces of evidence to an authentication mechanism: knowledge, possession, and inherence.

- Hashing: Hashing is the process of transforming any given key or a string of characters into another value. This is usually represented by a shorter, fixed-length value or key that represents and makes it easier to find or employ the original string. The most popular use for hashing is the implementation of hash tables. A hash function is any function that can be used to map data of arbitrary size to fixed-size values. The values returned by a hash function are called hash values, hash codes, digests, or simply hashes. The values are usually used to index a fixed-size table called a hash table.

- Encryption (Symmetric vs Asymmetric): Symmetric encryption uses a single key that needs to be shared among the people who need to receive the message while asymmetric encryption uses a pair of public key and a private key to encrypt and decrypt messages when communicating.

- Using a private key, we can find the public key, but the private key cannot be obtained using the public key. Even though asymmetric encryption provides more protection to the keys, they are comparatively slower than symmetric encryption.

Study Video: https://youtu.be/O4pJeXgOJDs?t=9420

## OSI Model:

The OSI Model is a 7-layer conceptual model for how to communicate between devices on a network. It is the foundation for everything networking.

The Essentials

OSI Model:

7 Layers:

In the OSI reference model, the communications between a computing system are split into seven different abstraction layers:

Physical, Data Link, Network, Transport, Session, Presentation, and Application. We’ll describe OSI layers “top down” from the application layer that directly serves the end user, down to the physical layer.

7. Application Layer

The application layer is used by end-user software such as web browsers and email clients. It provides protocols that allow software to send and receive information and present meaningful data to users. A few examples of application layer protocols are the Hypertext Transfer Protocol (HTTP), File Transfer Protocol (FTP), Post Office Protocol (POP), Simple Mail Transfer Protocol (SMTP), and Domain Name System (DNS).

6. Presentation Layer

The presentation layer prepares data for the application layer. It defines how two devices should encode, encrypt, and compress data so it is received correctly on the other end. The presentation layer takes any data transmitted by the application layer and prepares it for transmission over the session layer.

5. Session Layer

The session layer creates communication channels, called sessions, between devices. It is responsible for opening sessions, ensuring they remain open and functional while data is being transferred, and closing them when communication ends. The session layer can also set checkpoints during a data transfer—if the session is interrupted, devices can resume data transfer from the last checkpoint.

4. Transport Layer

The transport layer takes data transferred in the session layer and breaks it into “segments” on the transmitting end. It is responsible for reassembling the segments on the receiving end, turning it back into data that can be used by the session layer. The transport layer carries out flow control, sending data at a rate that matches the connection speed of the receiving device, and error control, checking if data was received incorrectly and if not, requesting it again.

3. Network Layer

The network layer has two main functions. One is breaking up segments into network packets and reassembling the packets on the receiving end. The other is routing packets by discovering the best path across a physical network. The network layer uses network addresses (typically Internet Protocol addresses) to route packets to a destination node.

2. Data Link Layer

The data link layer establishes and terminates a connection between two physically-connected nodes on a network. It breaks up packets into frames and sends them from source to destination. This layer is composed of two parts—Logical Link Control (LLC), which identifies network protocols, performs error checking and synchronizes frames, and Media Access Control (MAC) which uses MAC addresses to connect devices and define permissions to transmit and receive data.

1. Physical Layer

The physical layer is responsible for the physical cable or wireless connection between network nodes. It defines the connector, the electrical cable or wireless technology connecting the devices, and is responsible for transmission of the raw data, which is simply a series of 0s and 1s, while taking care of bit rate control.

### Advantages of OSI Model

The OSI model helps users and operators of computer networks:

- Determine the required hardware and software to build their network.

- Understand and communicate the process followed by components communicating across a network.

- Perform troubleshooting, by identifying which network layer is causing an issue and focusing efforts on that layer.

The OSI model helps network device manufacturers and networking software vendors:

- Create devices and software that can communicate with products from any other vendor, allowing open interoperability

- Define which parts of the network their products should work with.

- Communicate to users at which network layers their product operates – for example, only at the application layer, or across the stack.

## OSI vs. TCP/IP Model

The Transfer Control Protocol/Internet Protocol (TCP/IP) is older than the OSI model and was created by the US Department of Defense (DoD). A key difference between the models is that TCP/IP is simpler, collapsing several OSI layers into one:

- OSI layers 5, 6, 7 are combined into one Application Layer in TCP/IP

- OSI layers 1, 2 are combined into one Network Access Layer in TCP/IP – however TCP/IP does not take responsibility for sequencing and acknowledgement functions, leaving these to the underlying transport layer.

A public IP address identifies you to the wider internet so that all the information you're searching for can find you. A private IP address is used within a private network to connect securely to other devices within that same network. Each device within the same network has a unique private IP address.Apr 22, 2021

Public IP address: Private IP address

External (global) reach: Internal (local) reach

Not free: Free

Other important differences:

- TCP/IP is a functional model designed to solve specific communication problems, and which is based on specific, standard protocols. OSI is a generic, protocol-independent model intended to describe all forms of network communication.

- In TCP/IP, most applications use all the layers, while in OSI simple applications do not use all seven layers. Only layers 1, 2 and 3 are mandatory to enable any data communication.

- HTTP: Hyper Text Transfer Protocol: Port 80-

- TCP: TCP stands for Transmission Control Protocol a communications standard that enables application programs and computing devices to exchange messages over a network. It is designed to send packets across the internet and ensure the successful delivery of data and messages over networks.

- UDP: User Datagram Protocol (UDP) refers to a protocol used for communication throughout the internet. It is specifically chosen for time-sensitive applications like gaming, playing videos, or Domain Name System (DNS) lookups. In computer networking, the User Datagram Protocol is one of the core members of the Internet protocol suite. With UDP, computer applications can send messages, in this case referred to as datagrams, to other hosts on an Internet Protocol network.

- IP: IP is the main protocol within the internet layer of the TCP/IP. Its main purpose is to deliver data packets between the source application or device and the destination using methods and structures that place tags, such as address information, within data packets.

- MAC: Media Access Control (MAC) address is a physical address that works at the data link layer of the OSI model. A MAC address is a 48 or 64-bit address associated with a network adapter. In a local area network (LAN) or other network, the MAC (Media Access Control) address is your computer's unique hardware number. (On an Ethernet LAN, it's the same as your Ethernet address.)

- Ethernet: Ethernet is the traditional technology for connecting devices in a wired local area network (LAN) or wide area network (WAN). It enables devices to communicate with each other via a protocol, which is a set of rules or common network language. Ethernet is a family of wired computer networking technologies commonly used in local area networks, metropolitan area networks and wide area networks. It was commercially introduced in 1980 and first standardized in 1983 as IEEE 802.3.

- 802.11: IEEE 802.11 is part of the IEEE 802 set of local area network technical standards and specifies the set of media access control and physical layer protocols for implementing wireless local area network computer communication. 802.11 is an evolving family of specifications for wireless local area networks (WLANs) developed by a working group of the Institute of Electrical and Electronics Engineers (IEEE). There are several specifications in the family and new ones are occasionally added.

- All the 802.11 specifications use the Ethernet protocol and Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) for path sharing. The original modulation used in 802.11 was phase-shift keying (PSK). However, other schemes, such as complementary code keying (CCK), are used in some of the newer specifications. The newer modulation methods provide higher data speed and reduced vulnerability to interference.

- The 802.11 specifications are summarized in our IEEE 802 Wireless Standards: Fast Reference.

## HTTP

HTTP is used for accessing data and Web pages from a server. It is the language of the Internet, and is the protocol used for communicating at the Application layer.

### The Essentials

- HTTP: Hypertext Transfer Protocol (HTTP) is an application-layer protocol for transmitting hypermedia documents, such as HTML. It was designed for communication between web browsers and web servers, but it can also be used for other purposes. The Hypertext Transfer Protocol is an application layer protocol in the Internet protocol suite model for distributed, collaborative, hypermedia information systems. 1) HTML stands for HyperText Markup Language. It is a well-known markup language used for web page development. It is commonly used in webpage design. 1) HTTP stands for Hypertext Transfer Protocol. It is a great means of data communication for the World Wide Web.

- HTTPS Ports 443: Hypertext Transfer Protocol Secure is an extension of the Hypertext Transfer Protocol. It is used for secure communication over a computer network and is widely used on the Internet. In HTTPS, the communication protocol is encrypted using Transport Layer Security or, formerly, Secure Sockets Layer.

- SSL: Secure Socket Layer- Secure sockets layer (SSL) is a networking protocol designed for securing connections between web clients and web servers over an insecure network, such as the internet. A user connects to an SSL-enabled service such as a website. The user's application requests the server's public key in exchange for its own public key. ... When the user sends a message to the server, the application uses the server's public key to encrypt the message.

- TLS: Transport Layer Security- Transport Layer Security is a cryptographic protocol designed to provide communications security over a computer network. The protocol is widely used in applications such as email, instant messaging, and voice over IP, but its use in securing HTTPS remains the most publicly visible. Transport Layer Security (TLS) is an Internet Engineering Task Force (IETF) standard protocol that provides authentication, privacy and data integrity between two communicating computer applications.

## IP Addresses

In this video, we will review IPv4 addresses, subnets, and private/public IP addresses.

### The Essentials

- IP: TCP/IP defines five classes of IP addresses: class A, B, C, D, and E. Each class has a range of valid IP addresses. The value of the first octet determines the class. IP addresses from the first three classes (A, B and C) can be used for host addresses.

- Subnet: A subnetwork or subnet is a logical subdivision of an IP network. The practice of dividing a network into two or more networks is called subnetting. Computers that belong to the same subnet are addressed with an identical most-significant bit-group in their IP addresses.

## Network Types

In this video, we'll review the different types of networking.

### The Essentials

- PAN: Personal Area Network (Bluetooth)= A personal area network is a computer network for interconnecting electronic devices within an individual person's workspace. A PAN provides data transmission among devices such as computers, smartphones, tablets and personal digital assistants.

- LAN: Local Area Network= A local area network is a computer network that interconnects computers within a limited area such as a residence, school, laboratory, university campus or office building. By contrast, a wide area network not only covers a larger geographic distance, but also generally involves leased telecommunication circuits.

- WLAN: A wireless local area network (WLAN) is a wireless distribution method for two or more devices. WLANs use high-frequency radio waves and often include an access point to the Internet. A WLAN allows users to move around the coverage area, often a home or small office, while maintaining a network connection. CAN

- MAN: A metropolitan area network (MAN) is a computer network that connects computers within a metropolitan area, which could be a single large city, multiple cities and towns, or any given large area with multiple buildings. A MAN is larger than a local area network (LAN) but smaller than a wide area network (WAN)

- SAN: A storage area network or storage network is a computer network which provides access to consolidated, block-level data storage. SANs are primarily used to access data storage devices, such as disk arrays and tape libraries from servers so that the devices appear to the operating system as direct-attached storage. Local Area Network (LAN) and Storage Area Network (SAN) are the most used storage network system. LAN is a set of computers and peripheral devices that share a common communications link or wireless link to a server within a different geographic region. SAN provides high speed and is used for dedicated networks.

- WAN: In its simplest form, a wide-area network (WAN) is a collection of local-area networks (LANs) or other networks that communicate with one another. A wide area network is a telecommunications network that extends over a large geographic area. Wide area networks are often established with leased telecommunication circuits.

## VLAN

In this video, we'll review VLANs.

### The Essentials

- VLAN: A virtual LAN (VLAN) is a logical overlay network that groups together a subset of devices that share a physical LAN, isolating the traffic for each group. A LAN is a group of computers or other devices in the same place -- e.g., the same building or campus -- that share the same physical network.

## Firewalls

In this video, we'll review what a firewall is.

### The Essentials

- Firewall: Five types of firewall include the following:

- packet filtering firewall.

- circuit-level gateway.

- application-level gateway (aka proxy firewall)

- stateful inspection firewall.

- next-generation firewall (NGFW)

- ACL: In computer security, an access-control list is a list of permissions associated with a system resource. An ACL specifies which users or system processes are granted access to objects, as well as what operations are allowed on given objects. Each entry in a typical ACL specifies a subject and an operation.

- Host-based: A host-based firewall protects you from untrusted devices that are on the same network. It is particularly useful for mobile devices which are often carried around and connected to various networks especially when travelling.

- Network-based: Network firewalls are security devices used to stop or mitigate unauthorized access to private networks connected to the Internet, especially intranets. The only traffic allowed on the network is defined via firewall policies – any other traffic attempting to access the network is blocked.

## Linux.

Since the creation of Unix in the 1970s, a lot of operating systems have used it as their foundation. Many of these operating systems failed, while others succeeded.

Linux is one of the most popular Unix based operating systems. It's open source and is used all over the world across many industries.

One amazing feature of the Linux operating system is the Command Line Interface (CLI) which allows users to interact with their computer from a shell. The Linux shell is a REPL (Read, Evaluate, Print, Loop) environment where users can enter a command and the shell runs it and returns a result.

The ls command is one of the many Linux commands that allow a user to list files or directories from the CLI.

In this article, we'll go in depth on the ls command and some of the most important flags you'll need day-to-day.

Linux is a very popular operating system for large enterprises. Interacting with the command line is an important skill for any IT or software engineer.

The Essentials:

- PWD: The pwd Linux command prints the current working directory path, starting from the root ( / ). Use the pwd command to find your way in the Linux file system structure maze or to pass the working directory in a Bash script.

Print work directory

## pwd Command

The pwd command stands for print working directory. It is one of the most basic and frequently used commands in Linux. When invoked the command prints the complete path of the current working directory.

- LS: list files

The ls command is used to list files or directories in Linux and other Unix-based operating systems.

Just like you navigate in your File explorer or Finder with a GUI, the ls command allows you to list all files or directories in the current directory by default, and further interact with them via the command line.

- Launch your terminal and type ls to see this in action:

Ls –l= Display Directories/Folders Vertical

- CD: Change Directory: cd /tmp

- MV: Move Directory- The mv command moves files and directories from one directory to another, or renames a file or directory. If you move a file or directory to a new directory, it retains the base file name. When you move a file, all links to other files remain intact, except when you move it to a different file system.

- CP: You use the cp command for copying files from one location to another. This command can also copy directories (folders). The syntax of this command is: cp [... file/directory-sources] [destination] [file/directory-sources] specifies the sources of the files or directories you want to copy

- CHMOD: To change directory permissions in Linux, use the following:

- chmod +rwx filename to add permissions.

- chmod -rwx directoryname to remove permissions.

- chmod +x filename to allow executable permissions.

- chmod -wx filename to take out write and executable permissions.

chmod

Unix-like operating system command

### Description

In Unix and Unix-like operating systems, chmod is the command and system call used to change the access permissions and the special mode flags of file system objects. Collectively these were originally called its modes, and the name chmod was chosen as an abbreviation of change mode.

Most file systems include attributes of files and directories that control the ability of users to read, change, navigate, and execute the contents of the file system. In some cases, menu options or functions may be made visible or hidden depending on a user's permission level; this kind of user interface is referred to as permission-driven.

Two types of permissions are widely available: traditional Unix file system permissions and access-control lists (ACLs) which are capable of more specific control.

## Social Engineering

Learn about the basics of social engineering and common attack vectors.

The Essentials

- Phishing: the fraudulent practice of sending emails purporting to be from reputable companies in order to induce individuals to reveal personal information, such as passwords and credit card numbers. ("an email that is likely a phishing scam)- Phishing is a type of social engineering where an attacker sends a fraudulent message designed to trick a person into revealing sensitive information to the attacker or to deploy malicious software on the victim's infrastructure like ransomware.

- Physical Breach: A physical breach involves the physical theft of documents or equipment containing cardholder account data such as cardholder receipts, files, PCs, and POS systems. It can also be referred to as corporate espionage, and items at risk include: Laptop and Desktop Computers.

- Pretext Calling: In this context, a "pretext" phone call is simply a tape recorded telephone call between the victim and the suspect. The call is usually initiated by the victim, under the supervision of a law enforcement officer - preferably the lead investigator or case agent. The suspect is unaware that the call is being recorded.

## Malware

Learn about malware and common types of malware.

The Essentials

- Ransomware: Ransomware is a type of malware from cryptovirology that threatens to publish the victim's personal data or permanently block access to it unless a ransom is paid. While some simple ransomware may lock the system without damaging any files, more advanced malware uses a technique called cryptoviral extortion.

- Trojan: A Trojan Horse Virus is a type of malware that downloads onto a computer disguised as a legitimate program. The delivery method typically sees an attacker use social engineering to hide malicious code within legitimate software to try and gain users' system access with their software.

- Keylogger: Keystroke logging, often referred to as keylogging or keyboard capturing, is the action of recording the keys struck on a keyboard, typically covertly, so that a person using the keyboard is unaware that their actions are being monitored. Data can then be retrieved by the person operating the logging program.

- Rootkit: A rootkit is a malicious software bundle designed to give unauthorized access to a computer or other software. Rootkits are hard to detect and can conceal their presence within an infected system. Hackers use rootkit malware to remotely access your computer, manipulate it, and steal data. A rootkit is a collection of computer software, typically malicious, designed to enable access to a computer or an area of its software that is not otherwise allowed and often masks its existence or the existence of other software. The term rootkit is a compound of "root" and the word "kit".